PLASTIC RAIL JOINER FISHPLATE

“15mm LONG FINE SCALE”

SUITABLE FOR CODE 180 RAIL ONLY

A.S.A. Black Plastic - General Purpose Use

The Fishplate has been designed to be a tolerance fit and will require easing open to allow them to slide on the rail, this is easily done by gently prizing open with a small screwdriver as you fit them.  The bold head detail goes on the inside of the rails.  Allowance for expansion of the rails should be made when fitting the Fishplates.  The linier expansion of brass rail is .00001 inch per degree Fahrenheit, this must be multiplied by the difference between maximum (150˚f) (surface temperature, not air temperature) and minimum (20˚f) temperature outdoors in the U.K. which is about 130 degrees, multiplied by 36” gives us a measurement of 47 thou per yard (or 1.2 mm). So allow 1mm plus 1.2mm for 1 yard, 2.4mm for 2 yard and 3.6mm for 3 yard rail.  Now determine what temperature you are laying the track to give you your gap allowance.  Steel rail has 30% less expansion. Please make sure that any burr is removed from the rail ends before fitting.
